Lab 5

Camera calibration with OpenCV

Learning Objectives:

- To use OpenCV for camera calibration

- Detect chessboard corners for calibration.

- Compute intrinsic parameters and distortion coefficients.

- Apply image undistortion to remove lens distortion.

- Use calibration for real-world applications like AR and robotics.

Introduction:

Camera calibration is an essential process in computer vision that allows us to correct distortions caused by a camera lens and accurately map real-world points onto an image. Calibration estimates the intrinsic and extrinsic parameters that define how a camera perceives the environment. This process is crucial for applications such as augmented reality (AR), 3D reconstruction, autonomous navigation, and object tracking.

Generally when we have a scene and we want to project it into an image 3D to 2D we need to find the projection matrix which include the intrinsic and extrinsic parameters , finding this matrix is camera calibration

Why Do We Need Camera Calibration:

When an image is captured using a camera, distortions cause straight lines to appear curved, affecting depth estimation and object localization. Calibration corrects these distortions, ensuring accurate mapping of 3D world coordinates onto a 2D image plane.

How 3D scene become a 2D image

The final form of the projection Matrix

Camera Model: Intrinsic and Extrinsic Parameters:

A camera model consists of:

- Intrinsic Parameters (Internal camera properties):

  - Focal length (fx, fy): Defines how much the camera magnifies the scene.

  - Principal point (cx, cy): The image center where the optical axis meets the image plane.

  - Distortion coefficients (k1, k2, p1, p2, k3): Corrects radial and tangential distortions.

- Extrinsic Parameters (Defines camera position and orientation):

  - Rotation matrix (R): Describes how the camera is rotated.

  - Translation vector (T): Defines the camera’s position in space.

Mathematical Model: The relationship between 3D world coordinates  and 2D image points  is:

where K is the intrinsic matrix, and [R | T] represents the extrinsic parameters.

Steps of Camera Calibration:

📌 Step 1: Capture Chessboard Images

- Why? A checkerboard pattern provides well-defined corners, which help estimate distortion parameters.

- The images used for calibration must be taken from different angles to ensure accurate parameter estimation.

📌 Step 2: Detect Chessboard Corners

- Why? Detecting corner points provides a mapping between the real-world 3D structure and the 2D image.

- This function detects the corners required for calibration.

ret, corners = cv2.findChessboardCorners(gray, checkerboard_size, None)

📌 Step 3: Store Object Points and Image Points

- Why? Object points represent real-world 3D coordinates, and image points represent their corresponding 2D projections.

objpoints.append(objp)

imgpoints.append(corners)

- These lists store real-world and detected points for calibration.

📌 Step 4: Compute Camera Calibration Parameters

- Why? Computes intrinsic parameters (camera matrix) and distortion coefficients.

ret, mtx, dist, rvecs, tvecs = cv2.calibrateCamera(objpoints, imgpoints, gray.shape[::-1], None, None)

- mtx: Intrinsic matrix (focal length, principal point).

- dist: Distortion coefficients (radial and tangential distortion).

- rvecs:

- tvecs:

📌 Step 5: Apply Calibration to Remove Distortion

- Why? Corrects lens distortion and produces an undistorted image.

undistorted_img = cv2.undistort(img, mtx, dist, None, new_camera_mtx)

- cv2.undistort() removes distortions using the calibration parameters.

Lets do step 1 and 2

Task: Find Chessboard Corners for Calibration

import cv2

import numpy as np

# Load a checkerboard image

img = cv2.imread('checkerboard.jpg')

gray = cv2.cvtColor(img, cv2.COLOR_BGR2GRAY)

# Define chessboard size (columns, rows)

checkerboard_size = (6, 9)

# Find chessboard corners

ret, corners = cv2.findChessboardCorners(gray, checkerboard_size, None)

# Draw detected corners

if ret:

img = cv2.drawChessboardCorners(img, checkerboard_size, corners, ret)

cv2.imshow('Detected Corners', img)

cv2.waitKey(0)

cv2.destroyAllWindows()

Lets do step 3 and 4

### Task: Compute Camera Parameters

- Step 1: Different viewpoints of check-board image is captured.

- Step 2: Find Chessboard Corners for each image

- Step 3: Store Object Points and Image Points

- Step 4: given all previous information we can calculate the camera parameters using calibrateCamera() function

- Step 5: print the extrinsic and intrinsic parameters

Code Example:

import cv2

import numpy as np

import glob

import os

# Define the chessboard size (columns, rows)

checkerboard_size = (6, 9)

# Prepare object points

objp = np.zeros((checkerboard_size[0] * checkerboard_size[1], 3), np.float32)

objp[:, :2] = np.mgrid[0:checkerboard_size[0], 0:checkerboard_size[1]].T.reshape(-1, 2)

# Store object points and image points

objpoints = []  # Real world 3D points

imgpoints = []  # 2D points in image plane

# Define the path to the folder containing calibration images

image_folder = r'C:\Users\m.nasif\Desktop\CV\Lab 5\Images'

# Get all image file paths in the folder

image_files = glob.glob(os.path.join(image_folder, '*.jpg'))  # Change '*.jpg' to '*.png' if needed

# Process each image

for fname in image_files:

img = cv2.imread(fname)

if img is None:

print(f"Warning: Could not read {fname}")

continue

gray = cv2.cvtColor(img, cv2.COLOR_BGR2GRAY)

ret, corners = cv2.findChessboardCorners(gray, checkerboard_size, None)

if ret:

objpoints.append(objp)

imgpoints.append(corners)

# Perform camera calibration

ret, mtx, dist, rvecs, tvecs = cv2.calibrateCamera(objpoints, imgpoints, gray.shape[::-1], None, None)

# Print the camera matrix and distortion coefficients

print("Camera Matrix:\n", mtx)

print("Distortion Coefficients:\n", dist)

Exercise 1: Read image pattern from camera or folder and detect the pattern corners.

Exercise 2: Perform camera calibration using webcam and print the calibration parameters.

Bonus Task: Augmented Reality (AR) with Calibration

🔹 Objective:

Use camera calibration data to overlay a virtual object in a real-world scene.

🛠️ Task:

- Detect a chessboard pattern in real-time video.

- Correct distortions using the camera matrix.

- Overlay a 3D virtual object (cube or marker).

Reference:

- Open CV camera calibration

- Video Tutorials for the theoretical part